/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body STM32F4 小车 双协议(文本+0x42二进制)

******************************************************************************

* 协议规则：0x42帧头 + 地址 + 总长度 + 数据区 + 累加校验和

* 车控ADDR=1 总长度=10（1头+1地址+1长度+4字节速度+2字节舵机+1校验）

* 心跳ADDR=0 总长度=4

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

#define USB_FRAME_HEAD     0x42

#define USB_FRAME_LENMIN   4

#define USB_FRAME_LENMAX   12

#define USB_ADDR_CARCTRL   1

#define USB_ADDR_BUZZER    4

#define USB_ADDR_HEART     0

typedef struct

{

uint8_t recvStart;

uint8_t index;

uint8_t rxBuf[USB_FRAME_LENMAX];

}SerialFrame_t;

// 浮点/16位数据字节转换联合体

typedef union{

uint8_t buff[4];

float   f32;

int32_t i32;

}Bit32Union_t;

typedef union{

uint8_t buff[2];

uint16_t u16;

int16_t  i16;

}Bit16Union_t;

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

#define MAX_SPEED_PERCENT 100

#define PWM_TIM8_PERIOD   1600

#define PWM_TIM1_PERIOD   20000

#define SERVO_MIN_PULSE   500

#define SERVO_MAX_PULSE   2500

#define UART_TX_DELAY     100

#define STEER_INPUT_MIN  -50

#define STEER_INPUT_MAX   50

#define SERVO_ANG_LEFT_LIMIT  140

#define SERVO_ANG_RIGHT_LIMIT 40

#define SERVO_ANG_MID         90

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

TIM_HandleTypeDef htim1;

TIM_HandleTypeDef htim8;

UART_HandleTypeDef huart1;

/* USER CODE BEGIN PV */

// 文本M/T指令缓存

uint8_t rx_buffer[32];

uint8_t rx_index = 0;

uint8_t rx_data;

// 二进制帧解析结构体

SerialFrame_t serialFrame;

// 小车状态

int16_t current_speed = 0;

int16_t current_steer_input = 0;

uint16_t current_angle = 90;

uint16_t servoPwmSet = 1500;

// 电机驱动引脚定义

#define LEFT_MOTOR_IN1_PIN  GPIO_PIN_11

#define LEFT_MOTOR_IN2_PIN  GPIO_PIN_12

#define LEFT_MOTOR_IN1_PORT GPIOA

#define LEFT_MOTOR_IN2_PORT GPIOA

#define RIGHT_MOTOR_IN1_PIN  GPIO_PIN_8

#define RIGHT_MOTOR_IN2_PIN  GPIO_PIN_9

#define RIGHT_MOTOR_IN1_PORT GPIOC

#define RIGHT_MOTOR_IN2_PORT GPIOC

// PWM通道

#define LEFT_PWM_CHANNEL   TIM_CHANNEL_1

#define RIGHT_PWM_CHANNEL  TIM_CHANNEL_2

#define SERVO_PWM_CHANNEL  TIM_CHANNEL_1

// 上电提示

uint8_t init_ok_msg[] = "\r\n==== Car Init OK ====\r\nText Cmd:Mxx/Txx\r\nBinary Frame Head=0x42\r\nTest Frame:42 01 0A 00 00 00 00 DC 05 00\r\n";

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_TIM1_Init(void);

static void MX_TIM8_Init(void);

static void MX_USART1_UART_Init(void);

/* USER CODE BEGIN PFP */

void set_motor_speed(int16_t speed_percent);

void set_servo_steer(int16_t steer_val);

void set_servo_by_pwm(uint16_t pwmUs);

void process_command(char *cmd);

void SerialFrameParse(uint8_t byte);

void FrameDataHandle(void);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

void set_motor_speed(int16_t speed_percent)

{

if (speed_percent > MAX_SPEED_PERCENT) speed_percent = MAX_SPEED_PERCENT;

if (speed_percent < -MAX_SPEED_PERCENT) speed_percent = -MAX_SPEED_PERCENT;

uint16_t duty = (uint16_t)(abs(speed_percent) * (PWM_TIM8_PERIOD - 1) / MAX_SPEED_PERCENT);

if (speed_percent > 0)

{

HAL_GPIO_WritePin(LEFT_MOTOR_IN1_PORT, LEFT_MOTOR_IN1_PIN, GPIO_PIN_SET);

HAL_GPIO_WritePin(LEFT_MOTOR_IN2_PORT, LEFT_MOTOR_IN2_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN1_PORT, RIGHT_MOTOR_IN1_PIN, GPIO_PIN_SET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN2_PORT, RIGHT_MOTOR_IN2_PIN, GPIO_PIN_RESET);

}

else if (speed_percent < 0)

{

HAL_GPIO_WritePin(LEFT_MOTOR_IN1_PORT, LEFT_MOTOR_IN1_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(LEFT_MOTOR_IN2_PORT, LEFT_MOTOR_IN2_PIN, GPIO_PIN_SET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN1_PORT, RIGHT_MOTOR_IN1_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN2_PORT, RIGHT_MOTOR_IN2_PIN, GPIO_PIN_SET);

}

else

{

HAL_GPIO_WritePin(LEFT_MOTOR_IN1_PORT, LEFT_MOTOR_IN1_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(LEFT_MOTOR_IN2_PORT, LEFT_MOTOR_IN2_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN1_PORT, RIGHT_MOTOR_IN1_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(RIGHT_MOTOR_IN2_PORT, RIGHT_MOTOR_IN2_PIN, GPIO_PIN_RESET);

}

__HAL_TIM_SET_COMPARE(&htim8, LEFT_PWM_CHANNEL, duty);

__HAL_TIM_SET_COMPARE(&htim8, RIGHT_PWM_CHANNEL, duty);

current_speed = speed_percent;

}

void set_servo_steer(int16_t steer_val)

{

if (steer_val < STEER_INPUT_MIN) steer_val = STEER_INPUT_MIN;

if (steer_val > STEER_INPUT_MAX) steer_val = STEER_INPUT_MAX;

int32_t delta_input = steer_val - STEER_INPUT_MIN;

int32_t total_input_range = STEER_INPUT_MAX - STEER_INPUT_MIN;

int32_t total_angle_range = SERVO_ANG_LEFT_LIMIT - SERVO_ANG_RIGHT_LIMIT;

uint16_t angle = SERVO_ANG_LEFT_LIMIT - (delta_input * total_angle_range) / total_input_range;

if (angle > SERVO_ANG_LEFT_LIMIT) angle = SERVO_ANG_LEFT_LIMIT;

if (angle < SERVO_ANG_RIGHT_LIMIT) angle = SERVO_ANG_RIGHT_LIMIT;

uint16_t pulse = SERVO_MIN_PULSE + (uint16_t)((uint32_t)angle * (SERVO_MAX_PULSE - SERVO_MIN_PULSE) / 180);

if (pulse > PWM_TIM1_PERIOD) pulse = PWM_TIM1_PERIOD;

__HAL_TIM_SET_COMPARE(&htim1, SERVO_PWM_CHANNEL, pulse);

current_steer_input = steer_val;

current_angle = angle;

servoPwmSet = pulse;

}

void set_servo_by_pwm(uint16_t pwmUs)

{

if(pwmUs < SERVO_MIN_PULSE) pwmUs = SERVO_MIN_PULSE;

if(pwmUs > SERVO_MAX_PULSE) pwmUs = SERVO_MAX_PULSE;

servoPwmSet = pwmUs;

__HAL_TIM_SET_COMPARE(&htim1, SERVO_PWM_CHANNEL, pwmUs);

current_angle = (uint16_t)((uint32_t)(pwmUs - SERVO_MIN_PULSE) * 180 / (SERVO_MAX_PULSE - SERVO_MIN_PULSE));

}

void process_command(char *cmd)

{

while (*cmd == ' ') cmd++;

if (*cmd == '\0') return;

char echo_buf[128] = {0};

if (cmd[0] == 'M' || cmd[0] == 'm')

{

int16_t speed = atoi(cmd + 1);

set_motor_speed(speed);

uint16_t duty = abs(current_speed)*(PWM_TIM8_PERIOD-1)/100;

sprintf(echo_buf, "[TEXT OK] Speed=%d%% PWM=%d\r\n", current_speed, duty);

HAL_UART_Transmit(&huart1, (uint8_t *)echo_buf, strlen(echo_buf), UART_TX_DELAY);

}

else if (cmd[0] == 'T' || cmd[0] == 't')

{

int16_t steer_in = atoi(cmd + 1);

set_servo_steer(steer_in);

sprintf(echo_buf, "[TEXT OK] SteerIn=%d Angle=%d PWM=%dus\r\n", current_steer_input, current_angle, servoPwmSet);

HAL_UART_Transmit(&huart1, (uint8_t *)echo_buf, strlen(echo_buf), UART_TX_DELAY);

}

else

{

strcpy(echo_buf, "[ERR Cmd Use Mxx/Txx\r\n");

HAL_UART_Transmit(&huart1, (uint8_t *)echo_buf, strlen(echo_buf), UART_TX_DELAY);

}

}

/**

* @brief 二进制帧解析状态机（修正：rxBuf[2] 为总帧长）

*/

void SerialFrameParse(uint8_t byte)

{

// 注释掉所有调试打印

// char debugBuf[80];

// sprintf(debugBuf, "[DBG Byte:0x%02X]\r\n", byte);

// HAL_UART_Transmit(&huart1, (uint8_t *)debugBuf, strlen(debugBuf), UART_TX_DELAY);

uint8_t totalFrameLen;

if(!serialFrame.recvStart)

{

if(byte == USB_FRAME_HEAD)

{

serialFrame.recvStart = 1;

serialFrame.index = 0;

serialFrame.rxBuf[serialFrame.index++] = byte;

}

return;

}

if(serialFrame.index >= USB_FRAME_LENMAX)

{

serialFrame.recvStart = 0;

serialFrame.index = 0;

memset(serialFrame.rxBuf,0,USB_FRAME_LENMAX);

return;

}

serialFrame.rxBuf[serialFrame.index++] = byte;

if(serialFrame.index >= 3)

{

totalFrameLen = serialFrame.rxBuf[2];

if(totalFrameLen > USB_FRAME_LENMAX || totalFrameLen < USB_FRAME_LENMIN)

{

serialFrame.recvStart = 0;

serialFrame.index = 0;

memset(serialFrame.rxBuf,0,USB_FRAME_LENMAX);

return;

}

if(serialFrame.index >= totalFrameLen)

{

uint8_t checkSum = 0;

for(uint8_t i=0; i < totalFrameLen-1; i++)

checkSum += serialFrame.rxBuf[i];

if(checkSum == serialFrame.rxBuf[totalFrameLen-1])

{

FrameDataHandle();

}

else

{

// 保留错误提示

char debugBuf[80];

sprintf(debugBuf,"[ERR CheckSum Calc:0x%02X Rx:0x%02X]\r\n",checkSum,serialFrame.rxBuf[totalFrameLen-1]);

HAL_UART_Transmit(&huart1, (uint8_t *)debugBuf, strlen(debugBuf), UART_TX_DELAY);

}

serialFrame.recvStart = 0;

serialFrame.index = 0;

memset(serialFrame.rxBuf,0,USB_FRAME_LENMAX);

}

}

}

/**

* @brief 处理完整二进制帧

*/

void FrameDataHandle(void)

{

uint8_t addr = serialFrame.rxBuf[1];

char txBuf[128] = {0};

Bit32Union_t speedUnion;

Bit16Union_t servoUnion;

float speedMps;

int16_t motorPercent;

switch(addr)

{

case USB_ADDR_CARCTRL:

// 数据区从索引3开始，速度占4字节，舵机占2字节

memcpy(speedUnion.buff, &serialFrame.rxBuf[3], 4);

memcpy(servoUnion.buff, &serialFrame.rxBuf[7], 2);

speedMps = speedUnion.f32;

if(speedMps > 1.0f) speedMps = 1.0f;

if(speedMps < -1.0f) speedMps = -1.0f;

motorPercent = (int16_t)(speedMps * 100.0f);

set_motor_speed(motorPercent);

set_servo_by_pwm(servoUnion.u16);

sprintf(txBuf,"[BIN OK] Spd=%.2fm/s Motor=%d%% Servo=%dus Angle=%d°\r\n",

speedMps,motorPercent,servoUnion.u16,current_angle);

HAL_UART_Transmit(&huart1, (uint8_t *)txBuf, strlen(txBuf), UART_TX_DELAY);

break;

case USB_ADDR_HEART:

strcpy(txBuf,"[BIN Heart Recv]\r\n");

HAL_UART_Transmit(&huart1, (uint8_t *)txBuf, strlen(txBuf), UART_TX_DELAY);

break;

default:

sprintf(txBuf,"[BIN Unknown ADDR:%d]\r\n",addr);

HAL_UART_Transmit(&huart1, (uint8_t *)txBuf, strlen(txBuf), UART_TX_DELAY);

break;

}

}

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

memset(&serialFrame,0,sizeof(SerialFrame_t));

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_TIM1_Init();

MX_TIM8_Init();

MX_USART1_UART_Init();

/* USER CODE BEGIN 2 */

HAL_TIM_PWM_Start(&htim1, SERVO_PWM_CHANNEL);

HAL_TIM_PWM_Start(&htim8, LEFT_PWM_CHANNEL);

HAL_TIM_PWM_Start(&htim8, RIGHT_PWM_CHANNEL);

set_motor_speed(0);

set_servo_by_pwm(1500);

HAL_UART_Transmit(&huart1, init_ok_msg, sizeof(init_ok_msg)-1, UART_TX_DELAY);

// 开启单次中断接收

HAL_UART_Receive_IT(&huart1, &rx_data, 1);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief TIM1 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM1_Init(void)

{

/* USER CODE BEGIN TIM1_Init 0 */

/* USER CODE END TIM1_Init 0 */

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

TIM_BreakDeadTimeConfigTypeDef sBreakDeadTimeConfig = {0};

/* USER CODE BEGIN TIM1_Init 1 */

/* USER CODE END TIM1_Init 1 */

htim1.Instance = TIM1;

htim1.Init.Prescaler = 15;

htim1.Init.CounterMode = TIM_COUNTERMODE_UP;

htim1.Init.Period = 19999;

htim1.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim1.Init.RepetitionCounter = 0;

htim1.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_ENABLE;

if (HAL_TIM_Base_Init(&htim1) != HAL_OK)

{

Error_Handler();

}

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim1, &sClockSourceConfig) != HAL_OK)

{

Error_Handler();

}

if (HAL_TIM_PWM_Init(&htim1) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim1, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 0;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCNPolarity = TIM_OCNPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

sConfigOC.OCIdleState = TIM_OCIDLESTATE_RESET;

sConfigOC.OCNIdleState = TIM_OCNIDLESTATE_RESET;

if (HAL_TIM_PWM_ConfigChannel(&htim1, &sConfigOC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

sBreakDeadTimeConfig.OffStateRunMode = TIM_OSSR_DISABLE;

sBreakDeadTimeConfig.OffStateIDLEMode = TIM_OSSI_DISABLE;

sBreakDeadTimeConfig.LockLevel = TIM_LOCKLEVEL_OFF;

sBreakDeadTimeConfig.DeadTime = 0;

sBreakDeadTimeConfig.BreakState = TIM_BREAK_DISABLE;

sBreakDeadTimeConfig.BreakPolarity = TIM_BREAKPOLARITY_HIGH;

sBreakDeadTimeConfig.AutomaticOutput = TIM_AUTOMATICOUTPUT_DISABLE;

if (HAL_TIMEx_ConfigBreakDeadTime(&htim1, &sBreakDeadTimeConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM1_Init 2 */

/* USER CODE END TIM1_Init 2 */

HAL_TIM_MspPostInit(&htim1);

}

/**

* @brief TIM8 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM8_Init(void)

{

/* USER CODE BEGIN TIM8_Init 0 */

/* USER CODE END TIM8_Init 0 */

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

TIM_BreakDeadTimeConfigTypeDef sBreakDeadTimeConfig = {0};

/* USER CODE BEGIN TIM8_Init 1 */

/* USER CODE END TIM8_Init 1 */

htim8.Instance = TIM8;

htim8.Init.Prescaler = 0;

htim8.Init.CounterMode = TIM_COUNTERMODE_UP;

htim8.Init.Period = 1599;

htim8.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim8.Init.RepetitionCounter = 0;

htim8.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_ENABLE;

if (HAL_TIM_Base_Init(&htim8) != HAL_OK)

{

Error_Handler();

}

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim8, &sClockSourceConfig) != HAL_OK)

{

Error_Handler();

}

if (HAL_TIM_PWM_Init(&htim8) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim8, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 0;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCNPolarity = TIM_OCNPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

sConfigOC.OCIdleState = TIM_OCIDLESTATE_RESET;

sConfigOC.OCNIdleState = TIM_OCNIDLESTATE_RESET;

if (HAL_TIM_PWM_ConfigChannel(&htim8, &sConfigOC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

if (HAL_TIM_PWM_ConfigChannel(&htim8, &sConfigOC, TIM_CHANNEL_2) != HAL_OK)

{

Error_Handler();

}

sBreakDeadTimeConfig.OffStateRunMode = TIM_OSSR_DISABLE;

sBreakDeadTimeConfig.OffStateIDLEMode = TIM_OSSI_DISABLE;

sBreakDeadTimeConfig.LockLevel = TIM_LOCKLEVEL_OFF;

sBreakDeadTimeConfig.DeadTime = 0;

sBreakDeadTimeConfig.BreakState = TIM_BREAK_DISABLE;

sBreakDeadTimeConfig.BreakPolarity = TIM_BREAKPOLARITY_HIGH;

sBreakDeadTimeConfig.AutomaticOutput = TIM_AUTOMATICOUTPUT_DISABLE;

if (HAL_TIMEx_ConfigBreakDeadTime(&htim8, &sBreakDeadTimeConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM8_Init 2 */

/* USER CODE END TIM8_Init 2 */

HAL_TIM_MspPostInit(&htim8);

}

/**

* @brief USART1 Initialization Function

* @param None

* @retval None

*/

static void MX_USART1_UART_Init(void)

{

/* USER CODE BEGIN USART1_Init 0 */

/* USER CODE END USART1_Init 0 */

/* USER CODE BEGIN USART1_Init 1 */

/* USER CODE END USART1_Init 1 */

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USART1_Init 2 */

/* USER CODE END USART1_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOE_CLK_ENABLE();

__HAL_RCC_GPIOC_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_3|GPIO_PIN_11|GPIO_PIN_12, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_8|GPIO_PIN_9, GPIO_PIN_RESET);

/*Configure GPIO pins : PA3 PA11 PA12 */

GPIO_InitStruct.Pin = GPIO_PIN_3|GPIO_PIN_11|GPIO_PIN_12;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/*Configure GPIO pins : PC8 PC9 */

GPIO_InitStruct.Pin = GPIO_PIN_8|GPIO_PIN_9;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart)

{

if(huart->Instance == USART1)

{

// 二进制帧解析

SerialFrameParse(rx_data);

// 文本M/T行指令解析

if(rx_index >= sizeof(rx_buffer)-1)

{

rx_index = 0;

HAL_UART_Receive_IT(&huart1, &rx_data, 1);

return;

}

if(rx_data == '\r' || rx_data == '\n')

{

if(rx_index > 0)

{

rx_buffer[rx_index] = '\0';

process_command((char*)rx_buffer);

}

rx_index = 0;

}

else

{

rx_buffer[rx_index++] = rx_data;

}

// 核心修复：每次接收完成立刻重新开启中断

HAL_UART_Receive_IT(&huart1, &rx_data, 1);

}

}

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */